Practical 12

AIM: - a) Implement echo client server using TCP/UDP sockets.

Here’s a complete, easy-to-run example of an Echo Client-Server program using both TCP and UDP sockets in Python.

## A) TCP Echo Client-Server

### 🖥️ Server Code (tcp_server.py)

import socket

# Create TCP socket

server_socket = socket.socket(socket.AF_INET, socket.SOCK_STREAM)

server_socket.bind(('localhost', 12345))

server_socket.listen(1)

print("🚀 TCP Server is running and waiting for connection...")

conn, addr = server_socket.accept()

print("✅ Connected to:", addr)

while True:

data = conn.recv(1024).decode()

if not data:

break

print("📩 Received from client:", data)

conn.send(data.encode())  # Echo message back

conn.close()

print("🔚 Connection closed.")

### 💻 Client Code (tcp_client.py)

import socket

# Create TCP socket

client_socket = socket.socket(socket.AF_INET, socket.SOCK_STREAM)

client_socket.connect(('localhost', 12345))

while True:

msg = input("Enter message (type 'exit' to quit): ")

if msg.lower() == 'exit':

break

client_socket.send(msg.encode())

data = client_socket.recv(1024).decode()

print("🔁 Echo from server:", data)

client_socket.close()

### 🧩 Sample Input & Output

#### Server Output

🚀 TCP Server is running and waiting for connection...

✅ Connected to: ('127.0.0.1', 54213)

📩 Received from client: Hello Server

📩 Received from client: How are you?

📩 Received from client: exit

🔚 Connection closed.

#### Client Output

Enter message (type 'exit' to quit): Hello Server

🔁 Echo from server: Hello Server

Enter message (type 'exit' to quit): How are you?

🔁 Echo from server: How are you?

Enter message (type 'exit' to quit): exit

## ⚙️ B) UDP Echo Client-Server

### 🖥️ Server Code (udp_server.py)

import socket

server_socket = socket.socket(socket.AF_INET, socket.SOCK_DGRAM)

server_socket.bind(('localhost', 12346))

print("🚀 UDP Server is running...")

while True:

data, addr = server_socket.recvfrom(1024)

message = data.decode()

print(f"📩 Received from {addr}: {message}")

server_socket.sendto(data, addr)  # Echo message back

### 💻 Client Code (udp_client.py)

import socket

client_socket = socket.socket(socket.AF_INET, socket.SOCK_DGRAM)

server_addr = ('localhost', 12346)

while True:

msg = input("Enter message (type 'exit' to quit): ")

if msg.lower() == 'exit':

break

client_socket.sendto(msg.encode(), server_addr)

data, _ = client_socket.recvfrom(1024)

print("🔁 Echo from server:", data.decode())

client_socket.close()

### 🧩 Sample Input & Output

#### Server Output

🚀 UDP Server is running...

📩 Received from ('127.0.0.1', 53245): Hello via UDP

📩 Received from ('127.0.0.1', 53245): UDP works fine!

#### Client Output

Enter message (type 'exit' to quit): Hello via UDP

🔁 Echo from server: Hello via UDP

Enter message (type 'exit' to quit): UDP works fine!

🔁 Echo from server: UDP works fine!

Enter message (type 'exit' to quit): exit